Machine Run off and Acceptance.

1. Unpacking acceptance

Check the items in the boxes one by one according to the packing lists and the specific attachment list in the contract. Note on the bill of laden whether the packing box is intact, or the appearance of the machine tool is obviously damaged, does it have rust, peeling paint, or any damage.

2. Startup test

After the machine is installed. The main tests are as follows:

1) Various manual tests a. Manual movement of the machine. b. Jog test c. Spindle test d. Overtravel test

2) Function test a. Use buttons, switches, and manual operations to perform functional tests on the machine tool. b. Choose any spindle speed for test of spindle start, forward rotation, reverse rotation and stop. No less than 7 operations. c. Spindle high, medium and low speed conversion test. d. Choose a feed amount, and continuously do work feed and rapid feed tests on the XZ axis. e. Perform low, medium and high feed rate tests X and Z axes. f. The hydraulic and lubrication system checked g. The chuck is tested for clamping, loosening, flexibility and reliability. h. The spindle is tested for forward rotation, reverse rotation, stop and change of spindle speed. i. The turret tool post performs a forward and reverse rotation test if equipped. j. The feed mechanism is tested low, medium and high feed. k. Test the manual data input position display in DRO.

3) Idling test and running test of the active mechanisms should not be less than 1 hour. Continuous dry running test, the exercise time is not less than 4 hours, and each cycle time is not more than 15 minutes. Stop at the end of each cycle for no more than one minute before continuing to run.

4) The load test can be carried out in the following three steps: rough turning, heavy cutting and finishing turning. Each step is divided into single cutting and cycle. After each cutting is completed, the actual size of the processed part of the part is checked and compared with the DRO value.

3. Acceptance

After the machine tool is functionality tested the operation and maintenance training will begin